| NEC Electra Telephone System |  |
| --- | --- |
| Changing the time on a Electra Elite & Professional system must be done from the "main" or primary phone. It is usually extension # 100. |  |
| 1.- | Press "FEATURE" |
| 2.- | Dial "9#" |
| 3.- | Enter hour and minute |
| 4.- | Press "RECALL" to change AM/PM setting |
| 5.- | Press "HOLD" to change the calendar setting |
| 6.- | Press "RECALL" to change the day of week |
| 7.- | Dial "#" to move the cursor to day of month field |
| 8.- | Enter day of month |
| 9.- | Press "RECALL" to select month |
| 10.- | Dial "#"to move the cursor to year field |
| 11.- | Enter last 2 digits of year |
| 12.- | Press "FEATURE" |