Nortel Norstar Telephone Systems. Changing the time on a Norstar system must be done using a display phone. Using a M7310 or T7316 the right 2 buttons under the display are "NEXT" and "SHOW" respectively.. 1.-; To enter programming mode press "FEATURE" and "* * 2 6 6 3 4 4". 2.-; Enter password (default = 266344) - display should read "Terminals & Sets". 3.-; Press "NEXT" 5 times - display should read "Time and Date". 4.-; Press "SHOW" - display should read "Hour:___". 5.-; Press softkey "CHANGE". 6.-; Enter hour - display should read "AM OK or Change". 7.-; Select AM or PM. 8.-; Press "NEXT" - display should read "Minutes:___". 9.-; Press softkey "CHANGE". 10.-; Enter minutes. 11.-; Press "NEXT" - display should read "Year:___". 12.-; Press softkey "CHANGE". 13.-; Enter year. 14.-; Press "NEXT" - display should read "Month:___". 15.-; Press softkey "CHANGE". 16.-; Enter month. 17.-; Press "NEXT" - display should read "Day:___". 18.-; Press softkey "CHANGE". 19.-; Enter day. 20.-; Press "NEXT".

Nortel BCM Systems Time Change. The Nortel BCM systems automatically changes the time from the caller ID information. If the system is conneceted to a network it would get the time from the server..

Norstar Meridian CICS. Time. - Press "FEAT". - Press "*". - Press "*". - Press "266344" (CONFIG) or "23646" (ADMIN). - Enter password "23456" (ADMIN) or "72787" (SCPLUS). - You will see "Terminals & Sets". - Press "Next" five times. - Press "Show" - When you see "Hour", press "Change" or "Next" to skip. - Type in the hour using military time as a 2 digit number. - Press "Next". - When you see "Minutes", press "Change" or "Next" to skip. - Type in the minutes using a 2 digit number. - Press "Next".. Date:. - When you see "Year", press "Change" or "Next" to skip. - Type the year in 2-digit format. - Press "Next". - When you see "Month", press "Change" or "Next" to skip. - Type the month in 2-digit format. - Press "Next". - When you see "Day", press "Change" or "Next" to skip. - Type the day in 7-digit format. - Press "Next"..