# PANASONIC

Panasonic KX-TA624 / 824 Telephone System. Changing the time on a Panasonic KX-TA624 system must be done from extension # 101 using a display phone.. 1.-; Press the "PROGRAM" button on the face of the phone. 2.-; Enter programming mode by pressing "STAR" - "POUND" - "1234" (default password = 1234). 3.-; Enter program # 000. 4.-; Press "SPEAKER" button. 5.-; Press "FORWARD/DND" button to change to the hour selection. 6.-; Press "TRANSFER" button and enter the hour. 7.-; Press "FORWARD/DND" button to change to the minutes parameter. 8.-; Press "TRANSFER" button and enter the minutes. 9.-; Press "FORWARD/DND" button to change to the AM/PM parameter. 10; Press "MUTE" button to select AM or PM. 11.-; Press "STORE" button (listen for 1 tone if procedure was executed correctly / 3 tones if procedure failed). 12.-; Press "HOLD" button. 13.-; Press the "PROGRAM" button on the face of the phone.

Panasonic KX-TVS Voice Mail System. Changing the time on a Panasonic KX-TVS voice mail system can be done from any phone.. 1.-; Dial the TVS pilot number (extension 107 or 115). 2.-; When the voice mail system answers press "#" + "6" + "STAR". 3.-; Log into the message manager's mailbox (999). 4.-; Press "6" for other features. 5.-; Press "3" to set the time & date. 6.-; Press "1" to change the current time (press "0" for help). 7.-; Enter the hour then press "#" or enter the hour and minutes then press "#". 8.-; Press "1" for AM or "2" for PM. 9.-; Press "2" to accept the new time.

Panasonic Time Set KXT-61610 KSU. KX-T61610. 1.-; Use phone with intercom number 11. It will change the time for all phones.. 2.-; Keep handset down and press INTERCOM button.. 3.-; Dial 77.. 4.-; Dial the hour, with two digits (e.g. 03 for 3 o'clock).. 5.-; Dial the minutes, with two digits (e.g. 07 if it is now 7 minutes after the hour).. 6.-; Dial 0 for AM or 1 for PM.. 7.-; Press #.. 8.-; Press SP-PHONE.

Panasonic Digital Telephone System. Changing the time on a Panasonic digital system can be done from any display phone.. 1.-; Enter programming mode by pressing "PAUSE" then "STAR" button twice, followed by 1, 2, 3, 4. 2.-; Enter program # "000". 3.-; Press "SPEAKER" button. 4.-; Press "FORWARD/DND" button to change to the hour selection. 5.-; Press "TRANSFER" button and enter the hour. 6.-; Press "FORWARD/DND" button to change to the minutes parameter. 7.-; Press "TRANSFER" button and enter the minutes. 8.-; Press "FORWARD/DND" button to change to the AM/PM parameter. 9.-; Press "MUTE" button to select AM or PM. 10.-; Press "STORE" button. 11.-; Press "HOLD" button. 12.-; Lift and replace handset.

Panasonic DBS System. From the designated attendant console (usually x100, but not always), enter the following code sequence: To set the date and time:. 1.-; Press the ON/OFF key  The phone issues intercom dial tone.  The current date and time are displayed on your LCD.. 2.-; Press the PROG key. "F" appears on the display. 3.-; Dial "##." "Program Mode:" appears on the first line of the display; "MAIN MODE" appears on the second line.. 4.-; Press the FF1 key. "SELECT SUB MODE" appears on the second line of the display.. 5.-; Dial "1#." "1:DATE 2:TIME" appears.. 6.-; Dial "1#." "MONTH/DAY/YEAR" appears.. 7.-; Enter six digits for the month, day, and year. For example, to set the date to March 3, 1993, enter "030393.". 8.-; Press #. "TIME SET MODE appears on the display.. 9.-; Enter four digits for hours and minutes (24-hr. time). For example, for 11:00 a.m., enter "1100.". 10.-; Press #.. 11.-; Press the ON/OFF key.. The new date and time appear on the display.

Panasonic Digital Telephone System. Changing the time on a Panasonic KX-TD, KX-TDA or KX-TDE digital system can be done from any display phone.. 1.-; Enter programming mode by pressing "PROGRAM" button then "STAR" button twice, followed by 1, 2, 3, 4. 2.-; Enter program "000" and press Enter. 3.-; Press "SPEAKER" button two times, you will see the time.. 4.-; Enter the correct hour.. 5.-; Press "FORWARD/DND" button to advance to the minute selection. 6.-; Enter the correct minutes (if necessary to update).. 7.-; Press "FORWARD/DND" button to advance to the AM/PM selection.. 8.-; Press "MUTE" button to select AM or PM (if necessary to change).. 9.-; Press "STORE" button. 10.-; Lift and replace handset to exit the programming mode.. 11.-; This will change the time for all of the phones on your phone.

How to set the time on your Panasonic Voice Mail

- Press the intercom button on your phone. - Dial 165 and then raise the handset. - When the Voice Mail answers you, dial # 6 (POUND SIX) - Dial *999 (STAR NINE NINE NINE) - Enter your password. (usually your zip code) followed by the # (POUND) - Wait for the system to say, “To receive the message” - Then dial 6 (for other features) - Then dial 3 (to set the clock) - Then dial 1 (to change the time) - Enter the time followed by a # (POUND) Example for 9:37 is---Enter 937# - The system will ask for 1 for AM or 2 for PM - Enter the correct digit - The system will ask for 1 to change or 2 to accept - Enter 2 (to accept) - Hang up